CLI CODE (Using RIP V-2)

Main Campus Router

en conf t int gig0/0 no shut int se0/1/0 no shut int se0/1/1 no shut do wr exit int se0/1/1 clock rate 64000 int se0/1/0 clock rate 64000 do wr exit

Cloud Router

en conf t int gig0/0 no shut int se0/1/0 no shut do wr

Branch Campus Router

en conf t int gig0/0 no shut int se0/2/0 no shut do wr

Department Switch Config

ADMIN

en conf t int range fa0/1-24 switchport mode access switchport access vlan 10 do wr exit

HR

en conf t int range fa0/1-24 switchport mode access switchport access vlan 20 do wr exit

FINANCE

en conf t int range fa0/1-24 switchport mode access switchport access vlan 30 do wr exit

BUSINESS

en conf t int range fa0/1-24 switchport mode access switchport access vlan 40 do wr exit

E&C

en conf t int range fa0/1-24 switchport mode access switchport access vlan 50 do wr exit

A&D

en conf t int range fa0/1-24 switchport mode access switchport access vlan 60 do wr exit

STUD-LAB

en conf t int range fa0/1-24 switchport mode access switchport access vlan 70 do wr exit

IT-DEPT

en conf t int range fa0/1-24 switchport mode access switchport access vlan 80 do wr exit

STAFF

en conf t int range fa0/1-24 switchport mode access switchport access vlan 90 do wr exit

STUDENT LAB

en conf t int range fa0/1-24 switchport mode access switchport access vlan 100 do wr exit

L3 switch 1

en conf t int gig1/0/2 switchport mode access switchport access vlan 10 exit int gig1/0/3 switchport mode access switchport access vlan 20 exit int gig1/0/4 switchport mode access switchport access vlan 30 exit int gig1/0/5 switchport mode access switchport access vlan 40 exit int gig1/0/6 switchport mode access switchport access vlan 50 exit int gig1/0/7 switchport mode access switchport access vlan 60 exit int gig1/0/8 switchport mode access switchport access vlan 70 exit int gig1/0/9 switchport mode access switchport access vlan 80 exit do wr int gig 1/0/1 switchport mode trunk exit do wr

L3 switch 2

en conf t int gig1/0/3 switchport mode access switchport access vlan 90 exit int gig1/0/2 switchport mode access switchport access vlan 100 exit do wr int gig1/0/1 switchport mode trunk exit do wr

Main campus router

en conf t int se0/1/1 ip address 10.10.10.1 255.255.255.252 exit int se0/1/0 ip address 10.10.10.5 255.255.255.252 exit do wr int gig0/0.10 encapsulation dot1Q 10 ip address 192.168.1.1 255.255.255.0 exit int gig0/0.20 encapsulation dot1Q 20 ip address 192.168.2.1 255.255.255.0 exit int gig0/0.30 encapsulation dot1Q 30 ip address 192.168.3.1 255.255.255.0 exit int gig0/0.40 encapsulation dot1Q 40 ip address 192.168.4.1 255.255.255.0 exit int gig0/0.50 encapsulation dot1Q 50 ip address 192.168.5.1 255.255.255.0 exit int gig0/0.60 encapsulation dot1Q 60 ip address 192.168.6.1 255.255.255.0 exit int gig0/0.70 encapsulation dot1Q 70 ip address 192.168.7.1 255.255.255.0 exit int gig0/0.80 encapsulation dot1Q 80 ip address 192.168.8.1 255.255.255.0 exit do wr service dhcp ip dhcp pool admin-pool network 192.168.1.0 255.255.255.0 default-router 192.168.1.1 dns-server 192.168.1.1 exit service dhcp ip dhcp pool hr-pool network 192.168.2.0 255.255.255.0 default-router 192.168.2.1 dns-server 192.168.2.1 exit service dhcp ip dhcp pool finance-pool network 192.168.3.0 255.255.255.0 default-router 192.168.3.1 dns-server 192.168.3.1 exit service dhcp ip dhcp pool business-pool network 192.168.4.0 255.255.255.0 default-router 192.168.4.1 dns-server 192.168.4.1 exit service dhcp ip dhcp pool ec-pool network 192.168.5.0 255.255.255.0 default-router 192.168.5.1 dns-server 192.168.5.1 exit service dhcp ip dhcp pool ad-pool network 192.168.6.0 255.255.255.0 default-router 192.168.6.1 dns-server 192.168.6.1 exit service dhcp ip dhcp pool studlb-pool network 192.168.7.0 255.255.255.0 default-router 192.168.7.1 dns-server 192.168.7.1 exit service dhcp ip dhcp pool itdept-pool network 192.168.8.0 255.255.255.0 default-router 192.168.8.1 dns-server 192.168.8.1 exit do wr router rip version 2 network 10.10.20.0 network 10.10.10.4 network 192.168.1.0 network 192.168.2.0 network 192.168.3.0 network 192.168.4.0 network 192.168.5.0 network 192.168.6.0 network 192.168.7.0 network 192.168.8.0 exit do wr

Branch campus router

en conf t int se0/2/0 ip address 10.10.10.2 255.255.255.252 exit do wr int gig0/0.90 encapsulation dot1Q 90 ip address 192.168.9.1 255.255.255.0 exit int gig0/0.100 encapsulation dot1Q 100 ip address 192.168.10.1 255.255.255.0 exit do wr service dhcp ip dhcp pool staf-pool network 192.168.9.0 255.255.255.0 default-router 192.168.9.1 dns-server 192.168.9.1 exit do wr ip dhcp pool studlb-pool network 192.168.10.0 255.255.255.0 default-router 192.168.10.1 dns-server 192.168.10.1 exit do wr router rip version 2 network 192.168.9.0 network 192.168.10.0 network 10.10.10.0 exit do wr

Cloud router

en conf t int se0/1/0 ip address 10.10.10.6 255.255.255.252 exit int gig0/0 ip address 20.0.0.1 255.255.255.252 exit do wr router rip version 2 network 20.0.0.0 network 10.10.10.4 exit do wr

CLI CODE (Using  OSPF)

To remove the existing RIP (Each Router):

en conf t no router rip do wr

To impliment the OSPF Routing Protocol :

Main Campus Router

en conf t router ospf 1 router-id 1.1.1.1 network 192.168.0.0 0.0.255.255 area 0 network 10.10.10.0 0.0.0.3 area 0 network 10.10.10.4 0.0.0.3 area 0 exit do wr

Branch Campus Router

en conf t router ospf 1 router-id 2.2.2.2 network 192.168.9.0 0.0.0.255 area 0 network 192.168.10.0 0.0.0.255 area 0 network 10.10.10.0 0.0.0.3 area 0 exit do wr

Cloud Router

en conf t router ospf 1 router-id 3.3.3.3 network 10.10.10.4 0.0.0.3 area 0 network 20.0.0.0 0.0.0.3 area 0 exit do wr

CLI CODE (Using  EIGRP)

To remove the existing OSPF (Each Router) :

en conf t no router ospf 1 do wr

To impliment the OSPF Routing Protocol :

Main Campus Router

en conf t router eigrp 100 router-id 1.1.1.1 network 192.168.0.0 network 10.10.10.0 network 10.10.10.4 no auto-summary exit do wr

Branch Campus Router

en conf t router eigrp 100 router-id 2.2.2.2 network 192.168.9.0 network 192.168.10.0 network 10.10.10.0 no auto-summary exit do wr

Cloud Router

en conf t router eigrp 100 router-id 3.3.3.3 network 10.10.10.4 network 20.0.0.0 no auto-summary exit do wr